Tutorial For Investment Casting With Mould Poisoning

using IC-Mat CAFD code software

# Introduction:

The reaction of the material of the mould with liquid metal is sometimes unavoidable. Titanium and its alloys are very reactive; when casting these alloys it is necessary to consider the layer of impure material that will form at the surface. In this use of the simulation, a simple but expandable source term is added at the interface between metal and mould. The mould area needs to be specified by inputting an image in which the voxels are coded as mould or casting, or by coupling with a macro-model that can pass this information to the simulation routine.

Usage in this tutorial

Commands that you type are printed in:

courier 10 pt  (‘code’ style in the Document)

Computer commands enclosed in anglebrackets <> should be replaced with your own choice for file name, directory name etc. Do not type the anglebrackets.

File and directory names should not have contain spaces or other special characters.

# Getting started…

The IC-Mat CAFD microstructural simulation software is a cellular automaton growth model combined with finite-difference solution of diffusion equations. The program runs from a command line, reads in a series of input files that define the model, and produces several output files containing information about the simulated system at various points in time.

When combined with a macromodel, the behaviour of a large system may be simulated for averaged properties, while simulation of the microstructure is performed only on a small segment. This allows some insight into a system for which microstructural simulation of the whole would not be feasible.

## Prerequisites

- Familiarity with basic use of the computer system on which the model is being run is required, including creating, copying, and deleting files and directories, editing text files, and viewing image files. The code can run on a variety of systems, including but not limited to Gnu/Linux, Cygwin/Windows, SGI/Irix, and may be ported to other systems in the future. This is not intended to be a tutorial for Unix.

- You should already be able to run an IC-Mat CAFD job without macromodel coupling.

- In order to follow this tutorial, you should already be able to run a Procast job without IC-Mat CAFD. This is not meant to be a tutorial for Procast.

This tutorial was written for Procast 4.0.2. Please see the document Temporary instructions for CA_PROCAST for the changes required to use the current version of Procast.

## The Input Files

The following files are needed as input when using the coupled micro/macro model with Procast as the macromodel

| ca_ctrl.in | The main control files (possibly including alternate filenames for the other input files) |
| --- | --- |
| ca_geoplus.in | The geometry and boundary/initial conditions |
| ca_matprop.in | Global properties of the materials including nucleation model inputs |
| props_alloy0.in | Properties that apply only to the alloy solute |
| props_gas.in | Properties that apply only to the gas solute |
| ca_procast.in | Information required for transferring data between the micro and macro model including the location of the micromodel domain within the macromodel coordinates |

Each line in these input files consists of a Keyword followed by one or more Values. Any text beyond the required number of Values is ignored, so comments may be placed on the end of the line. Lines that begin with a hash mark (#) are also ignored and may be used for comments.

The following files are required, as always, for running the Procast macromodel:

| <jobname>p.dat |  |
| --- | --- |
| <jobname>d.dat |  |

These files are created by using Precast in the usual way. There are several other files produced during Precast, but only these final input files are necessary.

Example input files called crownp.dat and crownd.dat are provided with this tutorial. In this case the <jobname> is crown.

Please see the document Temporary instructions for CA_PROCAST for current information on how to run the new version. Features within the MatIc routines are not changed by the new version of Procast.

## Setting up the CAFD

- Make a directory within your jobs or tmp  directory where the simulation is going to be run. This will be called the working directory.

- Copy the executable and input files to the working directory.

- Copy the Procast input files <jobname>p.dat and <jobname>p.dat to the working directory

- Modify the input files.

- Control File: ca_ctrl.in

The mould source is activated by the keyword MouldSrc with value 1. Place the line

MouldSrc 1   /* Activate the mould source */

in the ca-ctrl.in file. This is a yes-or-no option where 0=no and 1=yes. If the value is 0 then the mould calculations are not done, if it is 1 they are done. Other values are not defined.  Ensure that there is no other line starting with this keyword.

| Table 1. Allowed inputs for the mould_src keyword in the props_ file |  |  |
| --- | --- | --- |
| Value | Internal Name | Description |
| 0 | NONE | No mould source for this component |
| 1 | CONST | Constant value at the interface |
| 2 | DIFF | Constant difference value (good for debugging not physically useful) |
| 3 | PLIN | Piecewise linear value, the segments are defined in the code. To change the line segments, modify or provide an alternate subroutine plin_mould_src and recompile |
| 4 | FLUX | Constant flux at the interface |

- Property files (props_alloy0.in, props_gas.in)

The model for the mould source is determined by the value of the keyword mould_src.This selects one model from a small number of different models available, shown in Table 1.

The relative strength of the mould source is determined by the value for the keyword mould_source_value:

mould_source_value 1.0

This is a multiplier applied to the source function, after the calculation by the chosen source model. Any floating point number may be used.

Zero or negative values have not been tested, if there is a meaningful physical reason to use these values you may have to carefully test or debug the code.

- Macromodel File: ca_procast.in

The location of the micromodel within the macromodel space is defined in the ca_procast.in file. Dimensions are in meters using the keyword BigBlockOrigin followed by three floating-point numbers:

BigBlockOrigin   -5.63e-3 -0.00701 -0.01193

The size of the micromodel domain is already fixed by the choice of cell size and number of cells, set up in the ca_geoplus.in control file.

Running the code:

The Master process for the coupled simulation using Procast is procast. This means that procast is run by the user, and procast calls the IC Mat CAFD code as a subroutine.

As a side effect of this, the user may not specify the name of the control file, it is fixed as ca_ctrl.in, and the macromodel interface file is fixed as ca_procast.in.

The job is run just as ordinary procast is run, starting from datacast

datacast <jobname>

procast <jobname>

On the  queue: [NOT YET AVAILABLE FOR PROCAST2006] There is a simple script provided that calls qsub and runs the procast job.

ca_pro_q  <job name>  <required time e.g 1:00:00 for 1 hour>

Users may wish to copy the scripts and modify for their own use, if they do not meet their needs. The scripts are /usr/local/bin/ca_pro_q (calling script) and /usr/local/bin/runcaprocast (execution script)

Queuing Jobs

Jobs in the queue may be monitored and manipulated by the commands provided by the torque version of OpenPbs queue manager software, qsub, qstat, qdel, qselect, and so on. Please read the manual pages accessed by

man qsub

man qstat

man qselect

man qdel

Most often, you will use qstat –an or qstat –s to check what is running, or what is not running.

## Results:

The usual results of procast may be post-processed by the procedures provided along with the Procast system.

The Mat CAFD code produces several output files at each selected execution step, and summary output files at the end of the run. When the jobs are run in the queue using runcaprocast, the output files are automatically collated and compressed into zip files.

### Compressed block files (.blz)

Compressed block files contain the complete state of the simulation at that time step. Any information in the simulation may be extracted from these files.

It could be possible to re-start a simulation from the stored time-point in the .blz file, if suitable mechanism for triggering this from the macromodel is used.

### Binary Files

Each binary output files contain the raw data for one variable in a selected 2-dimensional (x-y) slice of the micromodel domain. The file name is of the form

<FirstPrefix>_<SecondPrefix>_<basefilename>_<timepoint>.bin

The data type of the data is indicated by FirstPrefix, and the variable is indicated by SecondPrefix in the file name.

| FirstPrefix |  |
| --- | --- |
| F | floating point type (single or double depends on compilation) |
| I | integer type, e.g. grain number |
| SecondPrefix |  |
| A | Alloy solute |
| G | Gas solute |
| FS | Fraction Solid |
| UC | undercooling |

### Image Files

Image files of the selected 2-dimensional slice are output at each selected execution step. The files have the form

<basename>_<blocknum>t<timestep>.png

They may be viewed with any viewer that includes png(Portable network graphics) file capability or imported into documents.

### Post-processing

The data may be converted into a form suitable for importing into another software package. Scripts have been used to create image files and to import into TecPlot. The three-dimensional image files are simply ‘raw data’ and may easily be imported into VgStudioMax, Amira , ImageJ, Paraview, or Volview.

Data may be extracted from the compressed block .blz files by using the ca_read version of the Mat CAFD code. This is built from the same source code as ca_procast using the build command

makeca ca_read

You must use the same version as was used to build the ca_procast executable.

By specifying some options to ca_read, the raw data for that time step may be retrieved and optionally converted to single precision. The subblock number is always 0 for ca_procast results.

*************************************************

*   ./ca_read: Read output file from the CA model

*

*   Usage: ./ca_read -i input_file

*       The following command line options are allowed:

*       -i              -> specify input file

*       -b              -> specify subblock number and write a binary brick of bytes for that data

*       -q              -> quiet mode

*       -f              -> print compile-time options (cflags)

*       -v              -> print version information

*       -t              -> specify which data and type to write

*                          for selected -b block:

*                       f          float, fraction solid

*                       a          float, alloy component

*                       g          float, gas component

*                       u          float, calculated undercooling

*                       i          unsigned short int, grain number

*                       b          byte (unsigned char), grain number (mod 256)

*                       s          byte (unsigned char), solid (yes/no)

*                       p          byte (unsigned char), pores (yes/no)

*                       F          CA_FLOAT, fraction solid

*                       A          CA_FLOAT, alloy component

*                       G          CA_FLOAT, gas component

*                       U          CA_FLOAT, calculated undercooling

*                       I          int, grain number

*                          these options may be combined into a string, eg: -t FAb

*       -h              -> print this message

*

*************************************************