# Standard Operating Procedure (SOP) Title: Use of Solvent Towers

| Assessor: | Joshua Linfoot |  |  | Location of work: | MSRH 502A |
| --- | --- | --- | --- | --- | --- |
| Principal Investigator: |  |  | Prof Alan Spivey |  |  |
| Date of approval: |  | 13/09/2021 |  | Date for review: | 13/09/2022 |

## Justifying the hazards:

Solvent Towers consist of a column with alumina and pressurised system used to dispense dried solvents.

| Identify hazards with specific risk assessments and a College or a departmental approval process |  |  |  |
| --- | --- | --- | --- |
| Ionising radiation sources | ☐ | Biological sources (microorganisms, human/animal tissues, plants) | ☐ |
| Class 3R, 3B or 4 Lasers | ☐ | Offsite work | ☐ |
| Confirm if Lone working is permitted with this SOP? ☐ If it is permitted, describe the control measures for lone workers: |  |  |  |

## Preparing for the SOP:

- DO get trained by the responsible technician or Super-User before attempting to use solvent towers.

- DO prepare the collecting glassware (round bottom flask with neck 24mm and stopper) and carrier container with absorbent/blue roll in it.

- DON’T collect more than 2/3 of the volume of the flask.

## Procedure:

Start-up checks:

- All valves are closed.

- N2 ON at 45-50 psi.

- Check the glassware is free of cracks.

Dispensing of the solvent:

1. Turn Valve to N2 GAS. Attach flask with clip, support the flask and the connection.

2. Turn Valve to VACUUM and cycle N2 GAS/ VACUUM 3 times.

3. Turn Valve to Solvent. Slowly open Green (Flow) Tap to dispense solvent. Do not fill more than 2/3 full.

4. Close Green Tap and turn Valve to N2 GAS through CLOSED. Disconnect flask.

5. Turn Valve to CLOSED.

End checks:

- Hoses are capped.

- Valves are closed.

## Disposal:

Dispose of waste via the appropriate hazardous waste route.

## Personal Protective Equipment (PPE):

## Lab coat, safety specs, nitrile gloves.

## Risk Analysis of SOP and emergency procedures:

### (In addition to Safe Lab Practice)

### Always remember to include fire associated risks and control measures where appropriate

| Hazard | Raw risks | Current control measures | Residual risk (Low/Med/High) |
| --- | --- | --- | --- |
| Hazardous solvents | Exposure to chemicals via inhalation, skin or eye contact: intoxication, irritation. | Warning signs are attached informing on the hazards. PPE is required for work in the area. Additional gloves are available next to the towers. | Low |
|  | Spillages | Instruction, training and supervision is provided. Spill kit is available. |  |
| Nitrogen Gas | Leak | Regular leak test performed by the responsible technician. | Low |

| Additional control measures to minimise residual risks | Implementation date |
| --- | --- |

| Who may be harmed |  |
| --- | --- |
| Staff / students ☒ | Cleaners / Engineers ☐ |
| Supporting staff ☐ | Others (specify): |

Emergency procedures – describe the response(s) required by the user and lab members

Spillages – Use spill kit to absorb small spillages and dispose of the waste via hazardous route. In case of large spillage – evacuate, escalate (to the responsible technician) and report (SALUS). Leak or Continuous flow – Use N2 shut off valve and solvent shut off valve on the canister. Fire – If ignition occurs but extinction is managed in a controlled manner, ensure a SALUS report is completed at the earliest opportunity. If the fire is not controllable you must activate a fire alarm call point and evacuate. Inform Fire Safety Officers or Security where the fire is located and what it involves when they arrive at the building. Gas leak – report to the responsible technician.

Recommended trainings and records:

| List of individuals competent to demonstrate safe work practice and train others (level 1 trainers): | Names of those that have been trained and can work unsupervised (level 2) and date training completed: |
| --- | --- |