Chemistry Risk Assessment

Lab / Room No: 502 Building: Molecular Sciences Research Hub (MSRH), Chemistry Department, Imperial College London, White City Campus, London W12 0BZ. PI(s) in charge of area: Prof. Alan C. Spivey..

| What is the hazard(s) or hazardous activity? | Waste Solvent Drums & Containers - Use & Disposal of Contents |  |
| --- | --- | --- |
| Who may be affected? | Students and staff in lab Cleaners Maintenance staff Engineers Other - state who technicians |  |
| How will injury occur? | lifting drums/containers; dropping drums/containers; spillage/inhalation when transferring contents. |  |
| What is the likely outcome? | lifting injuries; cuts from broken glass; fire hazard; irritation/burns to eyes, skin and respiratory tract from waste solvent. |  |
| What control measures are in place? | two people to lift drums/bend knees; transport containers in carriers; wear gloves, lab coat and eye protection; use funnel when transferring contents; only fill drums to 85% capacity; place lids on drums/containers when not stored in ventilated area. |  |
| WHAT MIGHT HAPPEN? Tick all boxes that apply |  | WHAT EMERGENCY PROCEDURES ARE REQUIRED? |
| Clothing contamination |  | Remove and wash before re-use Other - please describe |
| Spillage in a fume hood |  | Describe procedure to follow below |
| a. Insignificant <50 ml: |  | Mop up with absorbent material and dispose of in hazardous waste |
| b. Significant >50 ml: |  | Prevent further spread by cordoning off pool with absorbent barriers, mop up with absorbent material and transfer to fumehood to allow excess solvent to safely evaporate, then dispose of in hazardous waste. |
| Spillage outside a fume hood |  | Describe procedure to follow below |
| a. Insignificant < 50 ml: |  | Mop up with absorbent material and dispose of in hazardous waste |
| b. Significant (i.e. will cause immediate harm) >50 ml: |  | Prevent further spread by cordoning off pool with absorbent barriers, mop up with absorbent material and transfer to fumehood to allow excess solvent to safely evaporate, then dispose of in hazardous waste. |
| Skin/eye contact: |  | Wash with copious amounts of running water for 20 mins Other - please describe |
| Cut/wound: |  | Encourage to bleed by washing with warm water. Then wash with copious amounts of running water for 20 mins. Report to Occupational Health with MSDS |
| Inhalation |  | Remove to fresh air Other |
| Fire: |  | Extinguisher Type – please describe CO2 Sand Other - please describe |
| Fume hood breakdown: |  | Turn off/remove from heat source Pull down sash Other – please describe place lid on container |
| Failure of other engineering control measure: |  |  |
| Other – Please give details |  |  |

I have read and understood this risk assessment, received the relevant training and will abide by the health and safety procedures for the above.

| Name | Date | Signature |
| --- | --- | --- |